DECREE OF THE MINISTER OF COMMUNICATION AND INFORMATION TECHNOLOGY OF THE REPUBLIC OF INDONESIA

NUMBER 34 YEAR 2015

ON

TECHNICAL REQUIREMENTS OF RADIO COMMUNICATION EQUIPMENT OF POINT-TO-POINT MICROWAVE LINK WITH DIGITAL HYBRID SYSTEM

BY THE GRACE OF GOD THE ALMIGHTY

MINISTER OF COMMUNICATION AND INFORMATION TECHNOLOGY OF THE REPUBLIC OF INDONESIA

Considering:      	a.  that in accordance with the provision of Article 71 paragraph (1) of 	the Government Regulation of the Republic of Indonesia Number 	52 Year 2000 on Telecommunication Provision, any 	telecommunication tool and equipment manufactured, assembled, 	imported for trade and / or used in the territory of the Republic of 	Indonesia shall fulfil the technical requirements;

`	b.	that based on consideration referred to in letter a,  it is deemed 	necessary to issue a Decree of the Minister of Communication 	and 	Information Technology on Technical Requirements 	of 	Radio Communication Equipment of Point-to-Point Microwave 	Link 	Digital With Digital Hybrid  System

Bearing in mind:  	1.  Law   of   the    Republic of Indonesia Number 36 Year 1999 on

Telecommunication     (State    Gazette   of    the     Republic      of Indonesia   Number 154 Year 1999,    Supplement to the State Gazette of the Republic of Indonesia Number 3881);

2.	Law of the Republic of Indonesia Number 39 Year 2008 on State Ministries of the Republic of Indonesia (State     Gazette   of    the    Republic    of Indonesia Number 166 Year 2008,   Supplement to  the State Gazette of the Republic of Indonesia Number 4916);

3.	Law of the Republic of Indonesia Number 30 Year 2014 on Governmental Administration 	(State Gazette   of    the    Republic  of Indonesia Number 292 Year 2014,   Supplement to  the State Gazette of the Republic of Indonesia Number 5601);

4.	Government Regulation of the Republic of Indonesia Number 52 Year 2000 on Provision of Telecommunication (State Gazette of the Republic of Indonesia Number 107 Year 2000, Supplement to the  State Gazette of the Republic of Indonesia Number 3980);

5.	Government Regulation of the Republic of Indonesia Number 53 Year 2000 on Use of Radio Frequency Spectrum and Satellite Orbit (State Gazette of the Republic of Indonesia Number 108 Year 2000, Supplement to the  State Gazette of the Republic of Indonesia Number 3981);

6.	Decree of the President of the Republic of Indonesia Number 54 Year 2015 on Ministry of Communication and Information Technology;

7..	Decree of the Minister of Communication and Information Technology Number 17/PER/M.KOMINFO/10/2010 on Organization and Work Method of the Ministry of Communication and Information Technology;;

8.	Decree of the Minister of Communication and Information Technology Number 5 Year 2013 Group of Telelcommunication Tools and Equipment;

9.	Decree of the Minister of Communication and Information Technology Number 18 Year 2014 on Certification of Telecommunication Tools and Equipment as amended by the Decree of the Minister of Communication and Information Technology Number 1 Year 2015 on Amendment to the Decree of the Minister of Communication and Information Technology Number 18 Year 2014 on Certification of Telecommunication Tools and Equipment;

10.	Decree of the Minister of Communication and Information Technology Number 25 Year 2014 on Table of  Allocation of Radio Frequency Spectrum;

11.	Decree of the Minister of Communication and Information Technology Number 15 Year 2015 on Guideline for the Composition of Technical Requirements of Tele communication Tools and Equipment;

12.	Decree of the Minister of Communication and Information Technology Number 33 Year 2015 on Planning of the Use of Radio Frequency Band of Point-to-Point Microwave Link. 	.

DECIDES :

To issue  :    DECREE   OF THE MINISTER OF COMMUNICATION AND INFORMATION TECHNOLOGY ON TECHNICAL REQUIREMENTS OF RADIO COMMUNICATION EQUIPMENT OF POINT-TO-POINT MICROWAVE LINK WITH DIGITAL HYBRID SYSTEM

Article 1

Any Radio Communication equipment of Point-to-Point Digital Microwave Link With Digital Hybrid System manufactured, assembled, imported for trade and / or used in the Territory of the Republic of Indonesia shall fulfil the technical requirements as included in the Attachment which is an inseparable part of this Ministerial Decree.

Article 2

Evaluation of the conformance of h obligation of any Radio Communication equipment of Point-To-Point Microwave Link with Digital Hybrid System referred to in Article 1 is conducted in accordance with the provision of legal regulation.

Article 3

This Ministerial Decree shall come into force on the date of its promulgation.

In order to make known to every body,  instruct the Promulgation of this Ministerial Decree by placing it in the State Announcement of the Republic of Indonesia.

Done at:               JAKARTA

On        : 31 December 2015

______________________

MINISTER OF COMMUNICATION AND INFORMATION TECHNOLOGY OF THE REPUBLIC OF INDONESIA,

Signed

RUDIANTARA

Promulgated at Jakarta

on 31 December  2015

DIRECTOR GENERAL OF LEGAL AFFAIRS

OF THE MINISTRY OF LAW AND HUMAN

RIGHTS OF THE REPUBLIC OF INDONESKIA

Signed

WIDODO EKATJAHJANA

STATE ANNOUNCEMENT OF THE PUBLIC OF INDONESIA YEAR 2015 NUMBER 2041

……….

For copy conform to the original

Ministry of Communication and Information Technology

Head of Bureau of Legal Affairs,

Signed

Bertiana Sari

ATTACHMENT

DECREE OF THE MINISTER OF COMMUNICATION AND INFORMATION TECHNOLOGY OF THE REPUBLIC OF INDONESIA

NUMBER 34 YEAR 2015

ON

TECHNICAL REQUIREMENTS OF RADIO COMMUNICATION EQUIPMENT OF POINT-TO-POINT MICROWAVE LINK WITH DIGITAL HYBRID SYSTEM

TECHNICAL REQUIREMENTS OF RADIO COMMUNICATION EQUIPMENT OF POINT-TO-POINT MICROWAVE LINK WITH DIGITAL HYBRID SYSTEM

Technical requirements of Radio Communication equipment of Point-To-Point Microwave Link with Digital Hybrid System cover:

CHAPTER I    : General Provisions;

CHAPTER II   : Technical Requirements;

CHAPTER III  : Testing.

CHAPTER I

GENERAL PROVISIONS

A.	Scope

Radio Communication Equipment  	of Point-To-Point Microwave Link with Digital Hybrid System in this Ministerial Decree covers the equipment of:

1.	Microwave Link;  and

2.	Studio to Transmitter Link for the purpose of providing broadcast television;

with Digital Hybrid  System.

B.	Definition

In this Ministerial Decree what is meant by:

1.	Radio Communication Equipment of Point-To-Point Microwave Link with Digital Hybrid System is radio communication equipment that has the function to transmit information from one station / point to station / other point  (point to point), used  at transmission link system to channel baseband signal in the form of Ethernet (IP) and Plesiochronous Digital Hierarchy (PDH), Synchronous Digital Hierarchy (SDH), or Asynchronous Transfer Mode  (ATM).

2.	Point-to-Point Radio Communication is communication provided by a link of a station /point to a station / other point.

3.	Microwave Link is point-to-point radio communication system through micro wave which, among other things, is used at telecommunication backbone system, and transmission link, nd has the function of transmitting information from a station  / pint to station / other point (point-to-point).

4.	Telecommunication Backbone is terrestrial radio communication used for big capacity (SDH STM-1).

5.	Transmission Link is terrestrial radio communication used for small and medium capacity.

6.	Studio Transmitter Link is point-to-point communication that links broadcasting station (studio) fro  a broadcasting institution to transmitter means and / or transmitter means to channel broadcast.

7.	Spurious Emission is radio wave emission outside determined bandwidth.

8.	Antenna is radio sub equipment that functions to transmit or receive a  radio frequency signal.

9.	Jitter is variation within a short period which is not a significant instant cumulative  of a digital signal from its ideal position at a time scale.

C.	Abbreviations

1.	ATM    	:  Asynchronous Transfer Mode

2.	BER	:  Bit Error Rate

3.	CBR	:  Constant Bit Rate

4.	CS	:  Channel Separation

5.	dB	:  Decibel

6.	dBm	:  Decibel milli

7.	EWS	:  Engineering Work Station

8.	GE	:  Gigabit Ethernet

9.	GUI	:  Graphical User Interface

10.	HDB3	:  High Density Bipolar 3

11.	IDU	:  Indoor Unit

12.	IP	:  Internet Protocol

13.	ODU	:  Outdoor Unit

14.	PDH	:  Plesiochronous Digital Hierarchy

15.	PSK	:  Phase-Shift Keying

16.	QAM	:  Quadrature amplitude modulation

17.	QPSK	:  Quadrature Phase-Shift Keying

18.	RIC	:  Radio Interface Capacities

19.	RSL	:  Receive Input Signal Level

20.	SDH	:  Synchronous Digital Hierarchy

21.	STM	:  Synchronous Transport Module

22.	TDM	:  Time-Division Multiplexing

23.	UBR	:  Unspecified Bit Rate

24.	VBR-rt	:  Variable Bit Rate - real time

CHAPTER II

TECHNICAL REQUIREMENTS

A.	CONFIGURATION

Figure 1. Simple Configuration of Point-To-Point Radio Communication

Equipment Via Microwave with Digital Hybrid System

B.	FUNCTION OF EQUIPMENT

Radio Communication Equipment of Point-To-Point Microwave Link with Digital Hybrid System must fulfil the operational function as follows:

1.	Adaptive Modulation or Static Modulation;

2.	Selectable Channel Bandwidth;

3.	Capacity upgradeable by using software;

4.	Automatic protection switch for radio system (optional);

5.	TDM/IP/ATM interface susceptible to configuration (TDM/IP/ATM  interface that can be configurational); and

6.	Automatic transmitter power control.

C.	MAIN CHARACTERISTIC

1.	Radio Frequency

Radio Communication Equipment of Point-to-Point Microwave Link with Digital Hybrid System can only be operated at radio frequency range defined in Table 1

.

Explanation on Table 1 below:

a.	Frequency Range refers to ITU-R recommendation of F series

b.	Allocation and frequency are defined in accordance with Table of Allocation

of Radio Frequency Spectrum of Indonesia;

c.	Channel Space / bandwidth refer to the provision of legal regulation concerning Planning of the Use of Radio Frequency Band of Point-To-Point  Microwave Link.

.:

-

Table 1. Radio Frequency Range for using Radio Communication Equipment of Point-to-Point Microwave Link with Digital Hybrid  System

| Radio Frequency Range |  |
| --- | --- |
| Band (GHz) | Frequency Span (MHz) |

2.	Transmitter Output Power: ≤ 32 dBm.

3.	Signal Processing

a.	To increase the performance of transmission channel, Radio Communication Equipment of Point-To-Point  Microwave Link with Digital Hybrid System must support:

1)	Advance Forward Error Correction (FEC); and

2)	Adaptive Dynamic Modulation (ADM).

b.	Signal Processing at least hs two (2) kinds of scalable modulation, out of the kinds of scalable modulation below:

1)	QPSK;

2)	8 PSK;

3)	16 QAM;

4)	32 QAM;

5)	64 QAM;

6)	128 QAM;

7)	256 QAM;

8)	1024 QAM; and / or

9)	2048 QAM.

4.	Radio Frequency Spectrum Mask

Radio frequency spectrum mask of Radio Communication Equipment of Point-To-Point Microwave Link with Digital Hybrid System refers to standard of ETSI 302 217-2-2, with spectral limitation in accordance with the respective bandwidths / channel space as explained in the following table and figure:

a.	Table of Spectral Limitation

1)	Spectrum mask of radio frequency for bandwidth / channel space of 1.75 MHz.

Table 2. Spectrum Mask of Radio Frequency for  Bandwidth / channel space of 1.75

MHz

| Spectral efficiency Class | Min. RIC Rate (Mbit/s) | Reference of Masking Form | K1 (dB) | f1 (MHz) | K2 (dB) | f2 (MHz) | K3 (dB) | f3 (MHz) | K4 (dB) | f4 (MHz) | K5 (dB) | f5 (MHz) | K6 (dB) | f6 (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Figure 2 | 1 | 0.85 | -23 | 1.05 | -23 | 1.7 | -45 | 3 |  |  |  |  |
| 2 | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4L | 4 | Figure 3 | 1 | 0.8 | -28 | 1.1 | -55(1) | 3.5(1) |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -50(2) | 3.1(2) |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -45(3) | 2.6(3) |  |  |  |  |  |  |
| (1) For equipment that has frequency range of 3 GHz - 17 GHz. Additionally, for equipment that has frequency range below 10 GHz, there exists other spectrum mask selection where its spectrum mask floor is added with - 60 dB. (2) For equipment that has frequency range of 17 GHz - 30 GHz. (3) For equipment that has frequency range above 30 GHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Spectral efficiency Class | Min. RIC Rate (Mbit/s) | Reference of Masking Form | Variation of Frequency Corner for - 60 dB floor |
| --- | --- | --- | --- |
| 4L | 4 | Figure 3 | K3 / f3 = -60 dB / 4 MHz |

2)	Spectrum Mask of radio frequency for bandwidth / channel space of 3.5 MHz.

Table 3. Spectrum Mask of Radio Frequency for Bandwidth /Channel Space of 3.5 MHz

| Spectral efficiency Class | Min. RIC Rate (Mbit/s) | Reference of Masking Form | K1 (dB) | f1 (MHz) | K2 (dB) | f2 (MHz) | K3 (dB) | f3 (MHz) | K4 (dB) | f4 (MHz) | K5 (dB) | f5 (MHz) | K6 (dB) | f6 (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Figure 2 | 1 | 1.7 | -23 | 2.1 | -23 | 3.4 | -45 | 6 |  |  |  |  |
| 2 | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4L | 8 | Figure 3 | 1 | 1.6 | -28 | 2.2 | -55(1) | 7(1) |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -50(2 | 6.2(2) |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 45(3 | 5.2(3) |  |  |  |  |  |  |
| (1) For equipment that has frequency range of 3 GHz - 17 GHz. Additionally, for equipment that has frequency range below 10 GHz, there exists other spectrum mask selection where its spectrum mask floor is added with - 60 dB. (2) For equipment that has frequency range of 17 GHz - 30 GHz. (3) For equipment that has frequency range above 30 GHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Spectral efficiency Class | Min. RIC Rate (Mbit/s) | Reference of Masking Form | Variation of Frequency Corner for - 60 dB floor |  |  |  |  |  |  |  |  |  |  |  |
| 4L | 8 | Figure 3 | K3 / f3 = -60 dB / 8 MHz |  |  |  |  |  |  |  |  |  |  |  |

3)	Spectrum Mask of radio frequency for bandwidth / channel space of 7 MHz.

Table 4. Spectrum Mask of Radio Frequency for Bandwidth /Channel Space of 7 MHz

| Spectral efficiency Class | Min. RIC Rate (Mbit/s) | Reference of Masking Form | K1 (dB) | f1 (MHz) | K2 (dB) | f2 (MHz) | K3 (dB) | f3 (MHz) | K4 (dB) | f4 (MHz) | K5 (dB) | f5 (MHz) | K6 (dB) | f6 (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | Figure 2 | 1 | 3.4 | -23 | 4.2 | -23 | 6.8 | -45 | 12 |  |  |  |  |
| 2 | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4L | 16 | Figure 3 | 1 | 3.2 | -28 | 4.4 | -55(1) | 14(1) |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -50(2) | 12.4(2) |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -45(3) | 10.4(3) |  |  |  |  |  |  |
| 4H | 24 | Figure 4 | 1 | 3 | -10 | 3.75 | -33 | 4.2 | -40 | 8.75 | -55(1) -50(2) -45(3) | 13.75(1) 12.075(2) 10.425(3) |  |  |
| 5L | 29 (ACCP) | Figure 6 | 1 | 3 | -10 | 3.625 | -32 | 3.875 | -36 | 4.25 | -45 | 10 | -55(1) | 13.5(1) |
| 5H | 34 (ACCP) |  |  |  |  |  |  |  |  |  |  |  | -50(2) | 11.75(2) |
| 6L | 39 (ACCP) |  |  |  |  |  |  |  |  |  |  |  | -45(3) | 10(3)(4) |
| (1) For equipment that has frequency range of 3 GHz - 17 GHz. Additionally, for equipment that has frequency range below 10 GHz, there exists other spectrum mask selection where its spectrum mask floor is added with - 60 dB. (2) For equipment that has frequency range of 17 GHz - 30 GHz. (3) For equipment that has frequency range above 30 GHz. (4) For equipment that has frequency range above 30 GHz. corner points 5 and 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Min. RIC Rate (Mbit/s) | Min. RIC Rate (Mbit/s) | Reference of Masking Form | Variation of Frequency Corner for - 60 dB floor |  |  |  |  |  |  |  |  |  |  |  |
| 4L | 16 | Figure 3 | K3 / f3 = -60 dB / 16 MHz |  |  |  |  |  |  |  |  |  |  |  |
| 4H | 24 | Figure 4 | K5 / f5 = -60 dB / 15.425 MHz |  |  |  |  |  |  |  |  |  |  |  |
| 5L, 5H, 6L | 29, 34, and 39 (ACCP) | Figure 6 | K6 / f6 = -60 dB / 15.25 MHz |  |  |  |  |  |  |  |  |  |  |  |

4)	Spectrum Mask of Radio Frequency for Bandwidth / Channel Space of 13.75 MHz - 15 MHz (nominal 14 MHz).

Table 5. Spectrum Mask of Radio Frequency for Bandwidth /Channel Space of 13.75

MHz - 15 MHz (nominal 14 MHz)

| Spectral efficiency Class | Min. RIC Rate (Mbit/s) | Reference of Masking Form |  | K1 (dB) | f1 (MHz) | K2 (dB) | f2 (MHz) | K3 (dB) | f3 (MHz) | K4 (dB) | f4 (MHz) | K5 (dB) | f5 (MHz) | K6 (dB) | f6 (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | Figure 2 |  | 1 | 6.8 | -23 | 8.4 | -23 | 13.6 | -45 | 24 |  |  |  |  |
| 2 | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4L | 32 | Figure 3 |  | 1 | 1.6 | -28 | 2.2 | -55(1) | 7(1) |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -50(2) | 6.2(2) |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -45(3) | 5.2(3) |  |  |  |  |  |  |
| 4H | 49 | Figure 4 |  | 1 | 6 | -10 | 7.5 | -33 | 8.4 | -40 | 17.5 | -55(1) -50(2) -45(3) | 27.5(1) 24.15(2) 20.85(3) |  |  |
| 5L | 58 (ACCP) | Figure 6 |  | 1 | 6 | -10 | 7.25 | -32 | 7.75 | -36 | 8.5 | -45 | 20 | -55(1) -50(2) -45(3) | 27(1) 23.5(2)) 20(3)(4) |
| 5H | 68 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6L | 78 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6H | 88 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | 98 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 107 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) For equipment that has frequency range of 3 GHz - 17 GHz. Additionally, for equipment that has frequency range below 10 GHz, there exists other spectrum mask selection where its spectrum mask floor is added with - 60 dB. (2) For equipment that has frequency range of 17 GHz - 30 GHz and for 8B class from 17 GHz up to 43.5 GHz. . (3) For equipment that has frequency range above 30 GHz. (4) For equipment that has frequency range above 30 GHz. corner points 5 and 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Spectral efficiency Class | Min. RIC Rate (Mbit/s) | Reference of Masking Form |  | Variation of Frequency Corner for - 60 dB floor |  |  |  |  |  |  |  |  |  |  |  |
| 4L | 32 | Figure 3 |  | K3/f3 = -60 dB / 32 MHz |  |  |  |  |  |  |  |  |  |  |  |
| 4H | 49 | Figure 4 | K5/f5 = -60 dB / 30.85 MHz |  |  |  |  |  |  |  |  |  |  |  |  |
| 5L, 5H, 6L, 6H, 7, 8 | 58, 68, 78, 88, 98, and 107 (ACCP) | Figure 6 | K6/f6 = -60 dB / 30.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |

5)	Spectrum Mask of Radio Frequency for Bandwidth / Channel Space of 27.5 MHz - 30 MHz (nominal 28 MHz).

Table 6. Spectrum Mask of Radio Frequency for Bandwidth / Channel Space of 27.5

MHz - 30 MHz (nominal 28 MHz)

| Spectral efficiency Class | Min. RIC Rate (Mbit/s) | Reference of Masking Form | K1 (dB) | f1 (MHz) | K2 (dB) | f2 (MHz) | K3 (dB) | f3 (MHz) | K4 (dB) | f4 (MHz) | K5 (dB) | f5 (MHz) | K6 (dB) | f6 (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 16 | Figure 2 | 2 | 12.8 | -23 | 16.4 | -23 | 25 | -45 | 45 |  |  |  |  |
| 2 | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4L | 64 | Figure 3 | 2 | 12.8 | -27 | 27 | -55(1) | 56(1) |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -50(2) | 49(2) |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -45(3) | 42(3) |  |  |  |  |  |  |
| 4H | 98 | Figure 4 | 2 | 12 | -10 | 15 | -33 | 16.8 | -40 | 35 | -55(1) -50(2) -45(3) | 55(1) 48.3(2) 41.7(3) |  |  |
| 5LA | 117 (ACAP) | Figure 5 | 2 | 12.5 | -10. | 15 | -32 | 17 | -35 | 20 | -45 | 40 | -55(1) -50(2) -45(3) | 54(1) 47(2)) 40(3)(4) |
| 5HA | 137 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6LA | 156 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6HA | 176 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7A | 196 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8A | 215 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5LB | 117 (ACCP) | Figure 6 | 2 | 12 | -10. | 14.5 | -32 | 15.5 | -36 | 17 | -45 | 40 |  |  |
| 5HB | 137 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6LB | 156 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6HB | 176 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7B | 196 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8B | 215 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) For equipment that has frequency range of 3 GHz - 17 GHz. Additionally, for equipment that has frequency range below 10 GHz, there exists other spectrum mask selection where its spectrum mask floor is added with - 60 dB. (2) For equipment that has frequency range of 17 GHz - 30 GHz and for 8A class and 8B class from 17 GHz up to 43.5 GHz. . (3) For equipment that has frequency range above 30 GHz. (4) For equipment that has frequency range above 30 GHz. corner points 5 and 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Spectral efficiency Class | Min. RIC Rate (Mbit/s) | Reference of Masking Form | Variation of Frequency Corner for - 60 dB floor |  |  |  |  |  |  |  |  |  |  |  |
| 4L | 64 | Figure 3 | K3/f3 = -60 dB / 63 MHz |  |  |  |  |  |  |  |  |  |  |  |
| 4H | 98 | Figure 4 | K5/f5 = -60 dB / 61.7 MHz |  |  |  |  |  |  |  |  |  |  |  |
| 5LA, 5HA, 6LA,’ 6HA, 7A, 8A | 117, 137, 156, 176, 196, 215 (ACAP) | Figure 5 | K6/f6 = -60 dB / 61 MHz |  |  |  |  |  |  |  |  |  |  |  |
| 5LB, 5HB, 6LB, 6HB, 7B, 8B , | 117, 137, 156, 176, 196, 215 (ACCP) | Figure 6 | K6/f6 = -60 dB / 61 MHz |  |  |  |  |  |  |  |  |  |  |  |

6)	Spectrum Mask of Radio Frequency for Bandwidth / Channel Space of 40 MHz.

Table 7.  Spectrum Mask of Radio Frequency for Bandwidth / Channel Space of 40

MHz

| Spectral efficiency Class | Min. RIC Rate (Mbit/s) |  | Reference of Masking Form |  | K1 (dB) |  | f1 (MHz) | K2 (dB) | f2 (MHz) | K3 (dB) | f3 (MHz) | K4 (dB) | f4 (MHz) | K5 (dB) | f5 (MHz) | K6 (dB) | f6 (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5LA | 168 (ACAP) |  | Figure 5 |  | 2 |  | 18 | -10. | 21.5 | -32 | 24.5 | -35 | 29 | -45 | 57 | -55(1) - | 77(1) |
| 5HA | 196 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6LA | 224 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6HA | 252 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7A | 280 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8A | 308 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5LB | 168 (ACCP) |  | Figure 6 |  | 2 |  | 17.2 | -10. | 20.8 | -32 | 22.5 | -36 | 24.5 | -45 | 57 |  |  |
| 5HB | 196 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6LB | 224 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6HB | 252 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7B 8B | 280 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 308 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) For all equipment of frequency range. Additionally, for equipment that has frequency range below 10 GHz, there exists other spectrum mask selection where its spectrum mask floor is added with - 60 dB. Note: equipment using bandwidth or channel space of 40 MHz is generally high capacity connection. Hence at this table, spectral efficiency class starts from 5A class. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Spectral efficiency Class |  | Min. RIC Rate (Mbit/s) |  | Reference of Masking Form |  | Variation of Frequency Corner for - 60 dB floor |  |  |  |  |  |  |  |  |  |  |  |
| 5LA, 5HA, 6LA,’ 6HA, 7A, 8A |  | 168, 196, 224, 252, 280, 308 (ACAP) |  | Figure 5 |  | K6/f6 = -60 dB / 87 MHz |  |  |  |  |  |  |  |  |  |  |  |
| 5LB, 5HB, 6LB, 6HB, 7B, 8B |  | 168, 196, 224, 252, 280, 308 (ACCP) |  | Figure 6 |  | K6/f6 = -60 dB / 87 MHz |  |  |  |  |  |  |  |  |  |  |  |

7)	Spectrum Mask of Radio Frequency for Bandwidth / Channel Space of 55 MHz - 60 MHz (nominal 56 MHz)

.Table 8. Spectrum Mask of Radio Frequency for Bandwidth / Channel Space of 55

MHz - 60 MHz (nominal 56 MHz)

| Spectral efficiency Class | Min. RIC Rate (Mbit/s) | Reference of Masking Form | K1 (dB) | f1 (MHz) | K2 (dB) | f2 (MHz) | K3 (dB) | f3 (MHz) | K4 (dB) | f4 (MHz) | K5 (dB) | f5 (MHz) | K6 (dB) | f6 (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 32 | Figure 2 | 2 | 25.6 | -23 | 32.8 | -23 | 50 | -45 | 90 |  |  |  |  |
| 2 | 64 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 96 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4L | 128 | Figure 3 | 2 | 25.6 | -27 | 34 | -55(1) | 112(1) |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -50(2) | 98(2) |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -45(3) | 84(3) |  |  |  |  |  |  |
| 4H | 196 | Figure 4 | 2 | 24 | -10 | 30 | -33 | 33.6 | -40 | 70 | -55(1) -50(2) -45(3) | 110(1) 96.6(2) 83.4(3) |  |  |
| 5LA | 235 (ACAP) | Figure 5 | 2 | 25 | -10. | 30 | -32 | 34 | -35 | 40 | -45 | 80 | -55(1) -50(2) -45(3) | 108(1) 94(2) 80(3)(4) |
| 5HA | 274 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6LA | 313 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6HA | 352 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7A | 392 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8A | 431 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5LB | 235 (ACCP) | Figure 6 | 2 | 24 | -10. | 29 | -32 | 31 | -36 | 34 | -45 | 80 |  |  |
| 5HB | 274 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6LB | 313 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6HB | 352 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7B | 392 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8B | 431 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |

| (1) For equipment that has frequency range of 3 GHz - 17 GHz. Additionally, for equipment that has frequency range below 10 GHz, there exists other spectrum mask selection where its spectrum mask floor is added with - 60 dB. (2) For equipment that has frequency range of 17 GHz - 30 GHz and for 8A class and 8B class from 17 GHz up to 43.5 GHz. . (3) For equipment that has frequency range above 30 GHz. (4) For equipment that has frequency range above 30 GHz. corner points 5 and 6 |  |  |  |
| --- | --- | --- | --- |
| Spectral efficiency Class | Min. RIC Rate (Mbit/s) | Reference of Masking Form | Variation of Frequency Corner for - 60 dB floor |
| 4L | 128 | Figure 3 | K3/f3 = -60 dB / 126.0 MHz |
| 4H | 196 | Figure 4 | K5/f5 = -60 dB / 123.4 MHz |
| 5LA, 5HA, 6LA,’ 6HA, 7A, 8A | 235, 274, 313, 352, 392, 431 (ACAP) | Figure 5 | K6/f6 = -60 dB / 122 MHz |
| 5LB, 5HB, 6LB, 6HB, 7 B, 8B | 235, 274, 313, 352, 392, 431 (ACCP) | Figure 6 | K6/f6 = -60 dB / 122 MHz |

8)	Spectrum Mask of Radio Frequency for Bandwidth / Channel Space of 112 MHz.

Table 9.   Spectrum Mask of Radio Frequency for Bandwidth / Channel Space of 112 						MHz

| Spectral efficiency Class | Min. RIC Rate (Mbit/s) | Reference of Masking Form | K1 (dB) | f1 (MHz) | K2 (dB) | f2 (MHz) | K3 (dB) | f3 (MHz) | K4 (dB) | f4 f4 (MHz) | K5 (dB) | f5 (MHz) | K6 (dB) | f6 (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 64 | Figure 2 | 2 | 51.2 | -23 | 65.6 | -23 | 100 | -45 | 180 |  |  |  |  |
| 2 | 128 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 191 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4L | 256 | Figure 3 | 2 | 51.2 | -27 | 68 | -55(1) | 196(1) |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 45(2)) - | 168(2) |  |  |  |  |  |  |
| 4H | 392 | Figure 4 | 2 | 48 | -10 | 60 | -33 | 67.2 | -40 | 140 | -55(1) -45(3) | 193.2(1) 166.8(3) |  |  |
| 5LA | 470 (ACAP) | Figure 5 | 2 | 50 | -10. | 60 | -32 | 68 | -35 | 80 | -45 | 160 | -55(1) 45(2)(3) | 188(1) 160(2)(3)) |
| 5HA | 548 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6LA | 627 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6HA | 705 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7A | 784 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8A | 862 (ACAP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5LB | 470 (ACCP) | Figure 6 | 2 | 24 | -10. | 29 | -32 | 31 | -36 | 34 | -45 | 80 |  |  |
| 5HB | 548 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6LB | 627 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6HB | 705 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7B | 784 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8B | 862 (ACCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) For equipment that has frequency range of 17 GHz - 30 GHz and for 8A class and 8B class from 17 GHz up to 43.5 GHz (2) For equipment that has frequency range above 30 GHz. (3) For equipment that has frequency range above 30 GHz. corner points 5 and 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

9)	Spectrum Mask of Radio Frequency for Bandwidth / Channel

Space of 125 MHz.

Table 10. Spectrum Mask of Radio Frequency for Bandwidth / Channel Space of 125 						MHz

| Spectral efficiency Class | Min. RIC Rate (Mbit/s) | Reference of Masking Form | K1 (dB) | f1 (MHz) | K2 (dB) | f2 (MHz) | K3 (dB) | f3 (MHz) | K4 (dB) | f4 (MHz) | K5 (dB) | f5 (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 71 | Figure 7 | 3 | 57.3 | -18 | - 70 | -23 | 112 | - 40 | 181.3 | -40 | 31`2.5 |
| 2 | 142 |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 212 |  |  |  |  |  |  |  |  |  |  |  |
| 4L | 284 | Figure 8 | 3 | 57.3 | - 25 | 74.5 | -40(2) | 157 | - 40 | 312.5 |  |  |
| 4H | 438 | Figure 9 | 3 | 55 | -10 | 67 | -28 | 74.5 | -43 | 174 | -43 | 31`2.5 |
| 5LA | 438 | Figure 10 | 3 | 55 | -10 | 67 | -31 | 75.8 | -45 | 174 | -45 | 31`2.5 |
| 5HA | 612 |  |  |  |  |  | -34 | 77 | -45 |  | -45 |  |
| 6LA | 700 |  |  |  |  |  | -37 | 78.2 | -45 |  | -45 |  |
| 5B | 438 | Figure 11 | 3 | 53.5 | -10 | 64.8 | -31 | 69 | -45 | 174 | -45 | 31`2.5 |
| 5HB | 612 |  |  |  |  |  | -34 | 69.5 | -45 |  | -45 |  |
| 6LB | 700 |  |  |  |  |  | -37 | 70 | -45 |  | -45 |  |

10)	Spectrum Mask of Radio Frequency for Bandwidth / Channel

Space of  N x 250 MHz.

.

Table 11. Table 10. Spectrum Mask of Radio Frequency for Bandwidth / Channel Space

of  N x 250 MHz

| Spectral efficiency Class | Min. RIC Rate (Mbit/s) | Reference of Masking Form | K1 (dB) | f1 (MHz) | K2 (dB) | f2 (MHz) | K3 (dB) | f3 (MHz) | K4 (dB) | f4 (MHz) | K5 (dB) | f5 (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | N X 142 | Figure 7 | 3 | N X 114.5 | -18 | N X 140 | -23 | Nx 224 | - 40(2) | N x 362.5 | - 40(2) | 31`2.5 |
| 2 | N X 285(7) |  |  |  |  |  |  |  |  |  |  |  |
| 3 | N X 425(7) |  |  |  |  |  |  |  |  |  |  |  |
| 4L | N X 570(7) | Figure 8 | 3 | N X 114.5 | - 25 | N X 149 | -40(2) | N x 314 | - 40(2) | (1) |  |  |
| 4H | N X 875 | Figure 9 | 3 | N X 110 | -10 | N X 134 | -28 | N x 149 | -43(3)) | Nx 348 | -43(3)) | (1) |
| 5LA | NX1050(7) | Figure 10 | 3 | N X 110 | -10 | N X 134 | -31 | N x 151 | -45(4) | Nx 348 | -45(4) | (1) |
| 5HA | N X 1225 |  |  |  |  |  | -34 | N x 154 | -45(5) |  | -45(5) |  |
| 6LA | N X 1400 |  |  |  |  |  | -37 | N x 156 | -45(6) |  | -45(6) |  |
| 5LB | NX1050(7) | Figure 11 | 3 | N X 107 | -10 | N X 129.5 | -31 | N x 138 | -45(4) | Nx 348 | -45(4) | (1) |
| 5HB | N X 1225 |  |  |  |  |  | -34 | N x 139 | -45(5) |  | -45(5) |  |
| 6LB | N X 1400 |  |  |  |  |  | -37 | Nx 140 | -45(6) |  | -45(6) |  |
| (1) For equipment that has bandwidth / channel space of ≤500 MHz, the value of CS x 2.5 bandwidth / channel space is ≥500 MHz, the value of CS x 1.5 + 500. (2) For equipment that has attenuation less than -40 + log (N) is not compulsory. (3) For equipment that has attenuation less than -43 + log (N) is not compulsory. (4) As for N ≥ 2, for equipment that has attenuation less than -46 log (N) is not compulsory. (5) As for N ≥ 3, for equipment that has attenuation less than -49 + log (N) is not compulsory. (6) As for N ≥ 6, for equipment that has attenuation less than -52 + log (N) is not compulsory. (7) For class 2 (N=4), class 3 (N=5), class 4 (N=2 and N=4), class 5LA and 5LB (N= 1, 2, 3), the minimum of RIC the value of which approaches 1 Gbit / s is also considered valid. NOTE 1 : N may vary from 1 up to 8, vde annex Ea for more detail. NOTE 2 : 10 log (N) is meant for eliminating the first decimal value. |  |  |  |  |  |  |  |  |  |  |  |  |

b.	Reference Figure of Spectrum Mask Form

The following Figure shows a typical curve form and format which represent the above table of Spectrum Mask.

For all spectrum masks, the upper limit of radio frequency spectrum is 2.5 x CS. Hereafter is the Figure of Radio frequency spectrum mask in conformance with the Segment.

1)	Reference of Spectrum Mask Form for Spectral Efficiency Class 1, 2, and 3 (Radio Frequency Range below 57 GHz)..

Figure 2. Reference of Spectrum Mask Form for Spectral Efficiency Class 1, 2,           and 3  (Radio Frequency Range below 57 GHz).

2)	Reference of Spectrum Mask Form for Spectral Efficiency Class 4L (Radio Frequency Range below 57 GHz).

..

Figure 3. Reference of Spectrum Mask Form for Spectral Efficiency Class 4L           (Radio Frequency Range below 57 GHz).

3)	Reference of Spectrum Mask Form for Spectral Efficiency Class 4H (Radio Frequency Range below 57 GHz).

Figure 4. Reference of Spectrum Mask Form for Spectral Efficiency Class 4H          (Radio Frequency Range below 57 GHz).

4)	Reference of Spectrum Mask Form for Spectral Efficiency Class 5LA, 5HA, 6LA, 6HA, 7A, 8A (Radio Frequency Range below 57 GHz).

Figure 5. Reference of Spectrum Mask Form for Spectral Efficiency Class 5LA, 5HA, 6LA, 6 HA,7A, 8A (Radio Frequency Range below 57 GHz).

5)	Reference of Spectrum Mask Form for Spectral Efficiency Class 5L, 5H, 6L,6H, 7, 8, 5LB, 5HB, 6LB, 6HB, 7B, 8B (Radio Frequency Range below 57 GHz).

Figure 6. Reference of Spectrum Mask Form for Spectral Efficiency Class 5L, 5H, 6L, 6 H,7, 8, 5LB, 5HB, 6LB, 6HB, 7B, 8B (Radio Frequency Range below 57 GHz).

6)	Reference of Spectrum Mask Form for Spectral Efficiency Class 1, 2 and 3 (Radio Frequency Range of 71 GHz - 76 GHz and 81 GHz - 86 GHz)

.

Figure 7. Reference of Spectrum Mask Form for Spectral Efficiency Class 1, 2, and 3 (Radio Frequency Range of 71 GHz - 76 GHz and 81 GHz - 86 GHz)

7)	Reference of Spectrum Mask Form for Spectral Efficiency Class 4L (Radio Frequency Range of 71 GHz - 76 GHz and 81 GHz - 86 GHz)

Figure 8. Reference of Spectrum Mask Form for Spectral Efficiency Class 4L (Radio Frequency Range of 71 GHz - 76 GHz and 81 GHz - 86 GHz)

8)	Reference of Spectrum Mask Form for Spectral Efficiency Class 4H (Radio Frequency Range of 71 GHz - 76 GHz and 81 GHz - 86 GHz)

Figure 9. Reference of Spectrum Mask Form for Spectral Efficiency Class 4H (Radio Frequency Range of 71 GHz - 76 GHz and 81 GHz - 86 GHz)

9)	Reference of Spectrum Mask Form for Spectral Efficiency Class 5LA, 5HA, 6LA, 6HA, 7A, 8A (Radio Frequency Range of 71 GHz - 76 GHz and 81 GHz - 86 GHz)

Figure 10.. Reference of Spectrum Mask Form for Spectral Efficiency Class 5LA, 5HA, 6LA, 6HA, 7A, 8A (Radio Frequency Range of 71 GHz - 76 GHz and 81 GHz - 86 GHz)

10)	Reference of Spectrum Mask Form for Spectral Efficiency 	Class 5LB, 5HB, 6LB, 6HB, 7B, 8B (Radio Frequency Range 	of 71 GHz - 76 GHz and 81 GHz - 86 GHz)

Figure 11.. Reference of Spectrum Mask Form for Spectral Efficiency Class 5LB, 5HB

,6LB, 6HB, 7B, 8B (Radio Frequency Range of 71 GHz - 76 GHz and 81

GHz - 86 GHz)

5.	RSL

a.	for Frequency Band of 5 GHz - 11 GHz  (Bandwidth / Channel Space of 30 MHz and 56 / 60 MHz)

Table 12. BER as a function of RSL for Frequency Band of 5 GHz (4 400 - 5 000 MHz) -

11 GHz [10 700 - 11 700 MHz] [Bandwidth / Channel Space of 30 MHz and

56 / 60 MHz].

NOTE: For channel space of 28 MHz 30 MHz or 56 MHz - 60 MHz, the class system of 5LB, 5HB, 6LB, 6HB, 7B and 8B, the above Limit is applicable when using equally good antenna port in even or odd number of channels, with channel space of 30 MHz or 60 MHz, where each of them is separated by the same polarization, arranged using external hybrid coupler of 3dB or using narrow band banching filters. in that condition the value of threshold BER is deducted by 1.5 dB.

b.	for Frequency Band of 5 GHz  11 GHz (Bandwidth / Channel Space of 40 MHz).

Table 13. BER as a function of RSL for Frequency Band of 5 GHz (4 400 - 5 000 MHz) -

11 GHz [10 700 - 11 700 MHz) (Bandwidth / Channel Space of 40 MHz)

Note: The above limit is applicable when using equally good antenna port in even and odd number of channels, with channel space of 30 MHz or 60 MHz, where each of them is separated by the same polarization, arranged using external hybrid couple of 3 dB or using narrow band banching filters.In that condition, the value of threshold BER is deducted by 1.5 dB.

c.	for Frequency Band of 13 GHz  and 15 GHz.

Table 14. BER as a function of RSL for Frequency Band of 13 GHz (12 750 -  13 250

MHz) and 15 GHz (14 400 -15 350 MHz)

Note: The above limit is applicable when using equally good antenna port in even and odd number of channels with channel space of 30 MHz or 60 MHz, where each of them is separated by the same polarization, arranged using external hybrid coupler of 3 dB or using narrow band banching filters .In that condition the value of threshold BER is deducted by 1.5 dB.

d.	for Frequency Band of 18 GHz

Table 15. BER as a function of RSL for Frequency Band of 18 GHz (17 700 -  19 700

MHz)

Note: The above limit is applicable when using equally good antenna port in even and odd number of channels , with channel space of 30 MHz or 60 MHz, where each of them is separated by the same polarization, arranged using external hybrid coupler of 3 dB or using narrow band banching filters .In that condition the value of threshold BER is deducted by 1.5 dB.

e.	for E System with system for RIC <100 Mbit / s

Table 16. BER as a function of RSL for E System [Radio Frequency of 21 GHz (21 200 -

23 600 MHz), and 26 GHz - 28 GHz (27 500 - 29 500 MHz)  with a system for

RIC < 100 Mbit / s.

Table 17. BER as a function of RSL for E System [Radio Frequency of 32 GHz (31 800 -

33 400 MHz) and 38 GHz (37 000 - 39 500 MHz)  with a system for RIC < 100 Mbit / s]

f.	for E System with system for RIC ≥100 Mbit / s

Table 18. BER as a function of RSL for E System [Radio Frequency of 21 GHz (21 200 -

23 600 MHz) and 26 GHz - 28 GHz (27 500 - 29 500 MHz)  with a system for RIC ≥ 100 Mbit / s]

Table 19. BER as a function of RSL for E System [Radio Frequency of 32 GHz (31 800 - 33 400 MHz) and 38 GHz (37 000 - 39 500 MHz) with a system for RIC ≥ 100 Mbit / s]

g.	for Ea  System

Table 20. BER as a function of RSL for Ea System [Radio Frequency of 71 GHz (71 000 -  76 000 MHz) up to 80 GHz (81 000 - 86 000 MHz)

NOTE: RIC is rounded at the multiplication of 1 Gbit / s rate also considered as valid value.

6.	Spurious Emission

a.	Spurious Power Maximum :  -65 dBm / 30 kHz

b.	Noise floor Maximum         :  -85 dBm / 30 kHz

7.	Physical Interface

Radio Communication Equipment of Point-To-Point Microwave Link with Digital Hybrid System must have:

a.	port interface Ethernet; and

b.	at least one of the following physical interfaces:

1)	Port Interface of 2048 kbps;

2)	Port Interface of SDH; or

3)	Port interface of ATM.

With the following technical provisions:

a.	Port Interface Ethernet:

1)	Capable of providing interface ethernet of 10 / 100 T Base and or  1000 T Base (GE) with the type of RJ45 or GE optical connector;

2)	Characteristic of ethernet is in accordance with IEEE 802.3 recommendation;

3)	Any data interface has the capability of full-duplex operation, automatic configuration and flow control;

4)	Any data interface must be capable of providing service with equalizationminimum latency (less than 10 ms); and

5)	Support jumbo frame minimum 1600 bytes (optional)..

.

b.	Port Interface of 2048 kbps

a.	Bit rate           	: 2048 kbps ± 50 ppm (± 102.4 		  bps)

b.	Code				:  HDB3

c.	Line impedance	: 120Ώ (balance)

d.	Nominal peak voltage	: 3 V

of a “mark”

e.	Peak voltage of a “space”	:  0 V ± 0.3 V

f.	Pulse width	:  244 ns ± 25 ns

g.	Ratio of the amplitudes  of	:  0.95 ~ 1.05

positive and negative

pulses at the centre of the

pulse interval

h.	Ratio of the widths  of	:  0.95 ~ 1.05

positive and negative

pulses at the nominal

half amplitude

i.	Line attenuation	:  ≥ 6 dB

equalization

j.	Frame structure 	:  1 frame consists of 32 time 	 	   slots. Time slot 16 is for 	  	 	   signalling and multi frame 	            	   a lignment word / signal , time 	    	  slot 0 for alarm and frame 	 	  alignment signal

k.	BER Test	:  Maximum 1 x  10-9. Duration 15 	   minutes

l.	Return Loss	:  In accordance with radio

frequency range

.	.	.

\Table 20. Return Loss 2048 Kbps

| Frequency range )kHz) | Return Loss (dB) |
| --- | --- |
| 51 up to 102 | 12 |
| 102 up to 2048 | 18 |
| 2048 up to 3072 | 14 |

c.	Port Interface of SDH

Interface of SDH using STM-1Electric, STM-1 Optical or STM-4, must fulfil the following  requirements:

1)	Port Interface 155520kbps (electrical)

2)	Bit rate                                     	:   155520 kbps ± 20 ppm (± 		    3111 bps)

3)	Code	:  CMI

4)	Line impedance  	:  5 W resistive (unbalance)

5)	Nominal peak-peak voltage	:  1 ± 0.1 V

6)	Nominal Pulse width 	:  6.43 ns

7)	The rising time of amplitude	:  ± 2ns

between 10% and 90%

measured from fixed

amplitude

8)	Transition timing tolerance	:  (refers to maon value 50% of 	  	   amplitude point from negative

transitions).

a)	Negative transitions	:  ± 0.1 ns.

b)	Positive transitions	:  ± 0.5 ns.

at interval boundaries

unit

c)	Positive transitions	:  ± 0.35 ns.

at mid interval

boundaries

9)	Return loss at port output	:  > 15 dB at frequency of 8 MHz 	   up tp 240 MHz.

10)	Specifications at port input	:  Maximum attenuation of 		   channel of 12.7 dB at

frequency of 78 MHz.

11)	Return loss	:  > 15 dB at frequency of  8

MHz up to 240 MHz

d.	Port Interface 155520 kbps (optical)

1)	Bit rate: 155520 kbps ± 20 ppm (± 3111 bps)

2)	Maximum requirements for transfer jitter parameter is 0.1 dB.

e.	Port Interface 622080 kbps (optical)

1)	Bit rate: 622080 kbps ± 20 ppm (± 12442 bps)

2)	Maximum requirements for transfer jitter parameter is 0.1 dB.

f.	Port Interface Optical G

For usage of  Gigabit Ethernet (GE) with optical interface, the characteristic of port must fulfil the requirements in the following table:

Table 21. GE Ethernet Port Interface Requirement

.

g.	Port Interface of ATM

Equipment must support the following features:

1)	Data encapsulation is compatible with RFC 1483 and RFC 1577..

2)	 Point to Point Protocol (PPP) is compatible with RFC 2364.

3)	Adaption layer of AAL2 and AAL5..

4)	UBR, CBR and VBR-rt.

When an equipment is connected to ATM network, it must have a compatible interface.

8.	Control System

Radio Communication Equipment of Point-To-Point Microwave Link with Digital Hybrid System must have  the following control and monitoring system:

a.	it has local and central management capability  planned to be user friendly in using GUI viewer or Web Based;

b.	it has back up and restore capability of data configuration;

c.	it covers the functions of network management system, at least as indicated hereunder:

1)	Fault alarm Management, alarm monitoring, fault detection and

fault localization;

2)	Configuration management, network configuration and inventory, configuration and service activation (provisioning);

3)	Performance management, performance data, and performance measurement;

4)	Security management and user and group access privileges management.

9.	Power Supply

a.	Capable of receiving volt : (220 ± 10%) Vac / (50 ± 6%) Hz; or

b.	Capable of receiving volt : (-48 ± 10%) Vdc.

D.	SUPPLEMENTARY CHARACTERISTIC

1	Raw Material

Radio Communication Equipment of Point-To-Point Microwave Link with Digital Hybrid System and its components must be made from strong, solid and high quality material , anti-corrosion anti-condensation.

2.	Construction

a.	The equipment components must:

1)	be neatly and well arranged at Printed Circuit Board/PCB for each module; and

2)	be perfect, well isolated, and no jumper is found using cable at Printed Circuit Board/PCB.

b	Wiring and cabling requirements

1)	wiring and cabling in equipment must be neat and good;

2)	in case integration between cards or modules need wiring and cabling, such wiring and cabling must use plug-in jack system and avoid soldering as far as possible;

3)	power supply cable f negative and positive volt must be differentiated and connecting in the equipment must be good and srong.

c.	EMC and safety: refer to SNI CISPR 22 and 24 standard.

3.	Environmental Condition

Radio Communication Equipment of Point-To-Point Microwave Link with Digital Hybrid System must be able to operate constantly at the following environmental condition:

a.	Temperature up to 55oC.

b.	Density of 5 up to 95%.

CHAPTER III

TESTING REQUIREMENT

Testing of Radio Communication Equipment of Point-To-Point Microwave Link with Digital Hybrid System is done in accordance with the provision of legal regulation.

MINISTER OF COMMUNICATION AND INFORMATION TECHNOLOGY OF THE REPUBLIC OF INDONESIA

Signed

RUDIANTARA